Arch-H-Gntlet-art - 12/23/13

"Archer's Half Gauntlet" by The Honorable Lord Jochen Schwalbe.

NOTE: See also the files: archery-SCA-msg, C-A-Basics-art, C-A-Handbook-art, CA-Hunt-Tips-art, c-archery-msg, armor-msg, gauntlets-msg, armorers-lst.

************************************************************************

NOTICE -

This article was submitted to me by the author for inclusion in this set of files, called Stefan's Florilegium.

These files are available on the Internet at: http://www.florilegium.org

Copyright to the contents of this file remains with the author or translator.

While the author will likely give permission for this work to be reprinted in SCA type publications, please check with the author first or check for any permissions granted at the end of this file.

Thank you,

Mark S. Harris...AKA:..Stefan li Rous

stefan at florilegium.org

************************************************************************

Archer's Half Gauntlet
by The Honorable Lord Jochen Schwalbe

This Half Gauntlet Pattern is a modification of a standard heavy Half Gauntlet for the specific purpose of use by combat archers. It is intended to be easy to make with a minimum of tools and expertise (I use split pin rivets for the whole thing), be light and easy to use with a bow.

Note the cuff is sized for a small wrist, should you have a larger hand simply extend the ends to fit. Keep in mind that you have to allow for padding!

Step 1

Trace the pieces in your preferred metal and cut out (you will be making a left and right hand so flip the pattern pieces as appropriate). Should you be using 1.2mm mild or stainless I strongly recommend rolling the back edge of the cuff. I usually chase a line down the middle for aesthetics. Drill holes where marked.

Step 2

Roll the cuff to the point where you can place a rivet on the back edge (I usually use a split pin leather rivet for this, In fact, I use split pin rivets for the whole Gaunt)

Step 3

Cut a strip of leather 30-40mm wide to a length that will fit snugly inside the cuff (use leather approximately 3mm thick). You may wish to sew the ends together to form a band, or strategically place it such that the cuff and the thumb plate hold the ends together. Mark Holes in this band, punch and rivet this band in place on the inside of the cuff. Most of the width of this band should be protruding from the cuff.

Step 4

Shape the wrist plate to fit the back of your hand (just roll it gently). Line the middle hole up with the chase line and punch holes in the leather band. Attach this piece with TWO rivets only, the third one, labeled "strap" will be done later!

Step 5

Shape (with some gentle dishing) the thumbplate and line it up such that it covers the back of your thumb when your hand is in place. Mark the leather band, punch holes and rivet on.

Step 6

Cut out 6mm closed cell foam to go on the inside of the thumbplate, wristplate and leather. Covering this in fabric is a really good idea since the foam lasts longer, breaths a bit and looks much nicer!

Step 7

Place the foam in the gaunt and cut straps to go over the palm of your hand and around your thumb. Use thin leather cut to a comfortable thickness (typically 20mm). Punch holes to line up with the holes in the metal.

Step 8

Rivet the palm and thumb straps in place, This is where you put in that last rivet to secure the band with the wrist plate. Glue the padding in place.

Enjoy your shooting.
Should you have any queries please do not hesitate to contact me via E-mail at tony_swallow at hotmail.com .

Your Humble Landsknecht

Jochen Schwalbe

------

Copyright 2010 by Tony Swallow. tony_swallow at hotmail.com. Permission is granted for republication in SCA-related publications, provided the author is credited.  Addresses change, but a reasonable attempt should be made to ensure that the author is notified of the publication and if possible receives a copy.

If this article is reprinted in a publication, please place a notice in the publication that you found this article in the Florilegium. I would also appreciate an email to myself, so that I can track which articles are being reprinted. Thanks. -Stefan.

<the end>