Movng-Targets-art - 6/16/17

"Making Moving Targets" by THL Deryk Archer.

NOTE: See also the files: Arrow-Inspect-art, arch-shoots-msg, Encourag-Arch-art, P-Arch-Target-art, Triangle-Targ-art, SCA-T-Archery-art, Arrow-Matchng-art.

************************************************************************

NOTICE -

This article was added to this set of files, called Stefan's Florilegium, with the permission of the author.

These files are available on the Internet at: http://www.florilegium.org

Copyright to the contents of this file remains with the author or translator.

While the author will likely give permission for this work to be reprinted in SCA type publications, please check with the author first or check for any permissions granted at the end of this file.

Thank you,

Mark S. Harris...AKA:..Stefan li Rous

stefan at florilegium.org

************************************************************************

First published in the June 15, 2017 issue of The Æthelmearc Gazette

Making Moving Targets

by THLord Deryk Archer

A moving target will add fun to your outdoor shoot. Imagine a pig or a deer running across your field.

Take two 4 foot long 2 x 4’s, drill two holes in the bottom of each board, and hammer in two very long nails. Next, take a set of bolt cutters and cut the heads off the nails. This turns them in the spikes. The spikes hold the boards up right so you can set this target up by yourself.

Next, run guy wires off the boards. Take a 50 foot piece of rope, double it and twist it to make the rope stronger so it will not sag when you pull the target across it.

Next, take a hard cardboard tube and cover it in camo duct tape. This’ll make the tube disappear into the background. Next, zip tie the wings to the tube and then zip tie the tube to the top of the pig. Then, take the twisted clothesline and run it through the tube and tie it to the upright 2x4s.

Run a guy wire from the nose of the pig back behind the firing line to the marshal. Pull the pig across the range to give the illusion of flight. The guy wire is over 100 feet long, allowing the marshal to call the line and pull the target at the same time.

At Hornwood’s Scarlet Guard Inn earlier this month, I ran a range just for fun with these targets. I made a flying pig, and everybody enjoyed the pun all day long.

------

Copyright 2017 by Joseph Barlow. <tiger7tlc at gmail.com>. Permission is granted for republication in SCA-related publications, provided the author is credited.  Addresses change, but a reasonable attempt should be made to ensure that the author is notified of the publication and if possible receives a copy.

If this article is reprinted in a publication, please place a notice in the publication that you found this article in the Florilegium. I would also appreciate an email to myself, so that I can track which articles are being reprinted. Thanks. -Stefan.

<the end>